# Wave Overtopping Prediction Using Neural Networks

This manual brings together overtopping theory, dimensional analysis, EurOtop context, neural-network methodology, recent machine-learning interpretation, and a full operator manual for the current Python implementation. Throughout the text, references are cited inline in the form requested by the user, for example (van der Meer et al., 2018) or (van Gent et al., 2007), and no bibliography section is included.

## 1. Purpose, scope, and how this manual should be used

This document has two equally important objectives. First, it provides a high-detail theoretical introduction to the prediction of mean wave overtopping discharge using empirical reasoning, dimensionless analysis, and neural-network or machine-learning models. Second, it provides a step-by-step operating manual for the exact workflow consisting of train.py as the computational backend and gui.py/gui.exe as the desktop interface.

The manual is written for engineers, analysts, and technically literate operators. It assumes that the reader needs not only a superficial explanation of where to click or what command to run, but also a strong understanding of what the model is predicting, which physical variables control the answer, how uncertainty should be interpreted, why extrapolation is dangerous, and how the Python implementation differs from the official EurOtop ANN tools described in the literature.

The practical scope is very specific. The workflow can be used for single-scenario prediction, batch prediction, model training and re-training, diagnostics generation, and range checking against the calibration domain. The GUI documentation included in the GUI documentation explicitly states these intended tasks and also makes clear that the GUI is aligned to a two-stage backend workflow composed of holdout validation followed by full-data refit. That distinction is crucial because the validation stage estimates generalisation quality, whereas the refit stage produces the saved production model that is later used for prediction.

This is not a manual for all overtopping methods ever published. Instead, it is a manual that places the current workflow into the broader overtopping framework represented by EurOtop and subsequent neural-network and machine-learning work. In practice, that means the text below repeatedly compares three layers of understanding: classical overtopping physics, official ANN/XGB overtopping tools from the literature, and the exact custom implementation in the project Python files.

How to use this manual in practice. A first-time user should read Sections 2 to 8 in sequence. An experienced operator who mainly needs procedures may jump directly to Sections 9 and 10. A reviewer who needs to verify whether a prediction is credible should focus on Sections 8, 11, and 12, plus Appendix E. A developer who intends to maintain or extend the workflow should pay particular attention to the sections describing target definition, engineered features, ensemble construction, holdout diagnostics, and full-data refit.

Warning. Reading rule for safe engineering use. Whenever the manual says “prediction”, read that as “model estimate conditioned on the calibration data and on the assumptions encoded in the inputs”. It does not mean physical truth. The more the input scenario departs from the training domain, the more the prediction becomes an extrapolation rather than an interpolation.

## 2. Executive technical summary

Wave overtopping is the mean or event-scale passage of water over the crest of a coastal defence, seawall, dike, revetment, rubble-mound breakwater, or vertical wall when wave run-up, impact, or splash exceeds the available crest freeboard. In practical design and assessment, the main scalar quantity is the mean discharge q, usually reported in m³/s/m or l/s/m.

Classical overtopping engineering expresses the process using a combination of wave conditions at the structure toe, geometry of the structure, roughness and permeability effects, and regime indicators such as the breaker parameter or surf similarity parameter. The EurOtop formulation for smooth slopes is often written in the generic form q/g Hm03=a exp​−b Rc/Hm0c, with regime-specific expressions for breaking and non-breaking waves (van der Meer et al., 2018).

Neural-network and machine-learning models are attractive because overtopping depends on a highly nonlinear interaction between hydraulic forcing and geometric configuration. A sufficiently broad and homogeneous database allows a model to learn relationships that are difficult to represent with a single simple closed-form equation, especially for composite, bermed, or otherwise complex structures (van Gent et al., 2007; Formentin et al., 2017; den Bieman et al., 2021).

The official EurOtop ANN is a specific reference point. It uses dimensionless physically motivated inputs and, in the 2018 description, derives uncertainty from the distribution of predictions from 500 neural networks, providing a mean and a 5%/95% confidence interval (van der Meer et al., 2018).

The train.py model in this project is not the official EurOtop ANN. It is a custom bagged multilayer perceptron ensemble implemented with scikit-learn MLPRegressor, median imputation, standardisation, early stopping, and engineered hydraulic features. It trains on log10(sq) and saves a model.joblib bundle plus diagnostics.json and plot outputs. By default it uses 10 bagged MLP models, hidden layers (192, 96, 48, 24), and a 15% holdout split.

A critical conceptual update must be stated explicitly. In the corrected revision of train.py, the internal target is defined as sq=q/gHm0,toe3. This is the standard non-dimensional overtopping-discharge scaling used in EurOtop-style practice, expressed here with the toe-based spectral wave height Hm0,toe. Therefore, when the script displays sq, it is now a physically standard dimensionless discharge coefficient rather than a non-standard custom scaling.

The GUI is a practical engineering shell around that backend. It supports immediate model loading, single-case prediction, batch prediction, model training/refresh, diagnostics inspection, calibration-range checking, persistent defaults, and CSV export. The GUI is designed so that a user can treat the workflow as a repeatable desktop tool rather than as a command-line-only program.

The safest way to use the current workflow is: train from a well-curated database; review diagnostics; check whether the target case is inside the observed training range; run prediction; inspect the spread indicators P05, P50, P95 and any range warning; and then decide whether the result is acceptable as a preliminary estimate or whether more rigorous project-specific analysis is required.

High-level comparison of conceptual layers used throughout this manual

- Classical overtopping physics

  - What it does: Explains why overtopping occurs and what variables dominate it.

  - Why it matters operationally: Provides engineering intuition and guards against blind trust in black-box predictions.

- Empirical formulae

  - What it does: Give direct equations for representative structure classes.

  - Why it matters operationally: Useful as a cross-check and as a baseline for reasonableness.

- Official EurOtop ANN/XGB tools

  - What it does: Demonstrate how data-driven methods were formalised in practice.

  - Why it matters operationally: Provide context, precedent, and cautionary limits.

- Uploaded custom workflow

  - What it does: Implements a specific bagged MLP model and a desktop GUI.

  - Why it matters operationally: This is the exact workflow the user will actually run.

## 3. Wave overtopping fundamentals

Wave overtopping is not a steady fluid discharge phenomenon. The underlying hydrodynamic process is strongly unsteady: individual waves run up the structure, some fail to reach the crest, some barely exceed it, and some produce large bursts of water over the crest. The commonly reported discharge q is therefore a mean discharge over time and per unit width, not a literal continuous sheet of water of constant thickness. EurOtop explicitly emphasises that overtopping can arise as green water, impact-driven throw over the crest, and splash-related processes, although practical prediction formulae usually compress that complexity into a mean discharge variable (van der Meer et al., 2018).

Why mean discharge became the central overtopping variable. Mean discharge is comparatively easy to measure in laboratory flumes and basins, easier to store in databases, and easier to use in design tolerability criteria than full time histories of overtopping velocities, thicknesses, and individual volumes. That historical measurability is one reason why the overtopping literature and later data-driven models are built so heavily around q.

The physical mechanism is a balance between forcing and resistance. The forcing side includes water level, significant wave height, wave period, obliquity, breaking regime, and depth-limited transformation before the structure. The resistance side includes crest freeboard, slope shape, toe and berm configuration, crest width, roughness, permeability, and the ability of the structure to dissipate or reflect wave energy. Overtopping increases when forcing rises or resistance falls. That sounds trivial, but it is the organising principle behind nearly all empirical equations and nearly all engineered machine-learning features.

Three principal structural families dominate the engineering framework. First, sloping dikes and embankment seawalls, where run-up on the slope is central. Second, armoured rubble-mound structures, where permeability, roughness, armour configuration, and berm effects become more influential. Third, vertical or very steep walls, where non-impulsive and impulsive impact regimes must be distinguished and where foreshore effects can drastically change the overtopping mechanism (van der Meer et al., 2018).

The concept of “the toe of the structure” is operationally essential. Overtopping inputs are not defined by offshore wave conditions alone. They must be defined at the toe region or at the point where the structure sees the transformed waves after shoaling, depth limitation, and breaking. This matters because the current workflow, like the broader EurOtop approach, uses Hm0,toe and Tm−1,0,toe as key inputs rather than offshore-only descriptors.

A mean overtopping value is meaningful only relative to use and consequence. A discharge that may be acceptable for a rough breakwater with no public access may be unacceptable for a promenade, railway, road, utility corridor, or flood defence line. In other words, overtopping prediction is only half of the engineering question. The other half is tolerability: what discharge is acceptable for people, vehicles, assets, and the structural system itself (van der Meer et al., 2018).

Principal structure families and overtopping interpretation

- Smooth sloping dike / seawall

  - Typical dominant controls: Run-up, freeboard, breaker regime, obliquity, roughness

  - Machine-learning implication: Relatively clear physics; empirical formulae are useful and should often be checked against ML estimates.

- Rubble-mound / armoured slope

  - Typical dominant controls: Permeability, roughness, berm effects, crest geometry, armour characteristics

  - Machine-learning implication: Data-driven methods are valuable because multiple interacting effects are difficult to collapse into one short equation.

- Vertical / steep wall

  - Typical dominant controls: Foreshore influence, impulsive versus non-impulsive attack, freeboard, relative depth

  - Machine-learning implication: Regime classification becomes critical; inappropriate extrapolation can be highly misleading.

## 4. Key hydraulic and structural variables

This section is foundational. A neural-network model can only be interpreted correctly if each input variable is understood physically. The project scripts use a compact set of base variables and then derive additional dimensionless and trigonometric features from them. That means the reader must understand both the engineering meaning of the base inputs and the mathematical role of the engineered variables.

Significant wave height Hm0. In the EurOtop framework the preferred wave height for overtopping is the spectral significant wave height Hm0, often written as Hm0=4m0, where m0 is the zeroth spectral moment. The manual explicitly distinguishes Hm0 from H1/3, noting that differences can arise in shallow water and that the overtopping equations are generally built around Hm0 at the toe (van der Meer et al., 2018).

Spectral period Tm−1,0. The preferred period in much overtopping work is the spectral period Tm−1,0=m−1/m0. This period gives more weight to the longer wave components than a simple mean period and is therefore well suited to run-up and overtopping work. The project scripts accept Tm−1,0,toe directly as an input, reflecting this established practice.

Deep-water wavelength based on Tm−1,0. A common derived quantity is Lm−1,0=g Tm−1,02/(2π). The train.py script computes Lm−1,0,toe using exactly this form. This conversion is not a minor detail: many dimensionless overtopping inputs use a length normalization by wavelength rather than by geometric length in metres alone.

Wave steepness. Wave steepness is classically a ratio of wave height to wavelength. The script computes a form of wave_steepness =Hm0/L0. That in turn feeds the square-root steepness and breaker-parameter-style features used by the model.

Breaker parameter / surf similarity ξ. The EurOtop definition is commonly written as ξm−1,0=tan(α)/Hm0/Lm−1,0. It is a compact regime indicator linking slope and wave steepness. Low values correspond more to breaking or plunging behaviour, while higher values correspond more to surging or non-breaking behaviour. The the script creates xi_m10_lower and xi_m10_upper by combining the lower and upper slope cotangents with square-root wave steepness.

Freeboard and crest geometry. Rc is crest freeboard; Ac is armour crest freeboard; Gc is crest width. These variables matter because overtopping is controlled by how much vertical and horizontal crest resistance exists after run-up has developed. The current workflow uses both raw values and normalized ratios such as Rc/Hm0, Ac/Hm0, Gc/Lm−1,0, and Gc/Hm0.

Toe and berm geometry. ht is toe water depth, Bt is toe width, B is berm width, and hb is berm water depth or berm level descriptor according to the data convention. In the current workflow, these are not merely retained as raw numbers; they are intentionally normalized by Hm0 or Lm−1,0 so the model can learn more transferable patterns across scales.

Obliquity β. The scripts accept β in degrees and also derive |β|, cos(β), and |sin(β)|. This is a good example of engineered learning design: the raw angle alone may not be the most convenient numerical representation for a multilayer perceptron, whereas trigonometric transforms encode directional behaviour more smoothly.

Roughness/permeability factor gf. The overtopping literature often uses influence factors to account for roughness and permeability effects. The script retains gf as a raw base input and also creates the interaction feature gf_cos_beta, which effectively couples roughness/permeability behaviour with the directional attack angle.

Slope representation through cotangents. The project files use cotad and cotau, representing the lower and upper slope cotangents. Since many overtopping formulae are written in terms of tan(α), the script internally reconstructs inverse cotangents using safe division. This allows it to form breaker-like features and other ratio features without requiring the user to enter slope angles directly.

Base input variables used directly by the current workflow

- m

  - Meaning: Foreshore slope cotangent in the CLI help of train.py

  - Role in script: Base input

  - Operational significance: Controls antecedent geometry and near-structure wave transformation context.

- b / β

  - Meaning: Wave obliquity in degrees

  - Role in script: Base input

  - Operational significance: Directional attack angle; also transformed to cos and sin-based features.

- h

  - Meaning: Water depth at the toe/front of the structure

  - Role in script: Base input

  - Operational significance: Sets local depth context and depth-limited behaviour.

- Hm0,toe

  - Meaning: Spectral significant wave height at the toe

  - Role in script: Base input

  - Operational significance: Primary wave-height scaling variable.

- Tm−1,0,toe

  - Meaning: Spectral mean period at the toe

  - Role in script: Base input

  - Operational significance: Period used to derive wavelength and regime indicators.

- ht

  - Meaning: Toe water depth

  - Role in script: Base input

  - Operational significance: Captures toe submergence and local geometric control.

- Bt

  - Meaning: Toe width

  - Role in script: Base input

  - Operational significance: Represents toe-geometry length scale.

- gf

  - Meaning: Roughness/permeability factor

  - Role in script: Base input

  - Operational significance: Dissipation and roughness control.

- cotad

  - Meaning: Lower slope cotangent

  - Role in script: Base input

  - Operational significance: Used to derive lower-slope breaker parameter.

- cotau

  - Meaning: Upper slope cotangent

  - Role in script: Base input

  - Operational significance: Used to derive upper-slope breaker parameter.

- B

  - Meaning: Berm width

  - Role in script: Base input

  - Operational significance: Horizontal berm control.

- hb

  - Meaning: Berm water depth / berm level descriptor

  - Role in script: Base input

  - Operational significance: Vertical berm control in the data model.

- Rc

  - Meaning: Crest freeboard

  - Role in script: Base input

  - Operational significance: First-order resistance to overtopping.

- Ac

  - Meaning: Armour crest freeboard

  - Role in script: Base input

  - Operational significance: Secondary crest elevation descriptor.

- Gc

  - Meaning: Crest width

  - Role in script: Base input

  - Operational significance: Crest length scale relevant to post-run-up passage.

## 5. Core overtopping formulae used as theoretical context

The train.py script does not directly calculate overtopping from closed-form empirical equations. It is a learned model. However, a serious manual must still present the main formulae that define the overtopping field, because these formulae explain why the chosen features make physical sense and provide a reasonableness benchmark for the outputs.

Generic overtopping form. EurOtop presents a generic overtopping discharge relation in a Weibull-like shape, commonly written as q/g Hm03=a exp​−b Rc/Hm0c for Rc≥0. This compact expression says that overtopping decays quickly with increasing relative freeboard, but not necessarily with a simple straight exponential law in all regimes (van der Meer et al., 2018).

Breaking and non-breaking smooth-slope formulae. For smooth slopes, EurOtop gives separate formulations for breaking and non-breaking wave attack. These equations are among the most influential reference expressions in the field and are repeatedly used as comparison baselines in later machine-learning studies.

Classical smooth-slope breaking-wave form (mean-value approach):

qg Hm03=0.023 tan(α)γb ξm−1,0exp​−2.7 Rcξm−1,0 Hm0 γb γf γβ γv1.3

Classical smooth-slope non-breaking / upper-limit form:

qg Hm03=0.09 exp​−1.5 RcHm0 γf γβ γ*1.3

Interpretation. These expressions say that overtopping increases with larger incident waves, lower freeboard, smoother surfaces, more favorable direction of attack, and more energetic run-up regimes. They also show why a learning model that receives Rc/Hm0, roughness descriptors, obliquity, wavelength-based ratios, and slope parameters is physically sensible rather than arbitrary.

Breaker parameter definition. A key regime variable is the surf similarity or breaker parameter:

ξm−1,0=tan(α)Hm0/Lm−1,0

with

Lm−1,0=g Tm−1,022 π

This parameter is central because it condenses both wave steepness and slope steepness. In train.py, the feature names xi_m10_lower and xi_m10_upper are clear descendants of this logic, computed separately for the lower and upper slopes from cotad and cotau.

Representative vertical-wall context. For vertical and steep walls, the overtopping regime must distinguish non-impulsive from impulsive attack. Under non-impulsive conditions, the discharge can be described by an exponential freeboard decay. Under impulsive conditions, especially at higher relative freeboard, a power-law form appears because large, violent impact-related overtopping can persist even for high freeboards (van der Meer et al., 2018).

Representative vertical-wall context formulas from EurOtop:

Non-impulsive design / assessment form:

qg Hm03=0.062 exp​−2.61 RcHm0

Impulsive high-freeboard mean-value form:

qg Hm03=0.0014 Hm0h sm−1,00.5RcHm0−3

valid for Rc/Hm0≥1.35.

Why these equations still matter in a machine-learning manual. Even if the final predictor is a neural network, these equations tell the user what trends should usually be expected: increasing freeboard should generally reduce overtopping, smoother or less dissipative conditions should generally increase it, different structural families may require different regime logic, and extreme geometries deserve caution. Whenever an ML prediction contradicts those first-order expectations without a convincing data-based explanation, the result deserves scrutiny.

Formula roles in the present manual

- q/g Hm03

  - Why it appears here: Classical dimensionless overtopping discharge

  - How it connects to the project Python model: Benchmark for physical interpretation; this is now the same square-root scaling used internally in train.py.

- Lm−1,0=g T2/(2π)

  - Why it appears here: Toe-based wavelength scaling

  - How it connects to the project Python model: Computed directly in train.py as Lm−1,0,toe.

- ξm−1,0

  - Why it appears here: Breaker / regime indicator

  - How it connects to the project Python model: Approximated by engineered lower and upper slope features.

- Rc/Hm0, Ac/Hm0, Gc/L

  - Why it appears here: Relative freeboard and crest ratios

  - How it connects to the project Python model: Computed directly as engineered ratios in train.py.

- Roughness and obliquity factors

  - Why it appears here: Capture dissipation and direction effects

  - How it connects to the project Python model: Represented by gf, β, cos_beta, sin_beta_abs, and gf_cos_beta.

## 6. Why neural networks and machine learning are used for overtopping

Overtopping is a classic nonlinear multivariable engineering problem. The response is not governed by a single independent variable. Instead, it emerges from a coupled set of geometric and hydraulic descriptors: wave height, period, local depth, slope, toe and berm geometry, crest configuration, roughness/permeability, and angle of attack. Many of these variables interact multiplicatively rather than additively. For that reason, overtopping has been a natural candidate for data-driven models for many years.

The historical rationale is simple. Empirical formulae are excellent when the structure family is simple and the assumptions behind the formula are respected. But real structures are often composite, bermed, multi-slope, or otherwise too complicated to compress cleanly into one short equation with a few coefficients. Neural networks were introduced to learn from large overtopping databases and thereby interpolate across a broader space of geometries than traditional formulae handle comfortably (van Gent et al., 2007; Verhaeghe et al., 2008; Formentin et al., 2017).

A machine-learning model does not create physics from nothing. It maps patterns already present in the database. Therefore its usefulness depends on at least four conditions. First, the database must be large enough. Second, the database must be reasonably consistent and homogeneous. Third, the inputs must encode the physically meaningful variables. Fourth, the target case should lie near the domain represented by the training data. If any of these fail, model output quality deteriorates quickly.

The EurOtop manual itself explicitly recognises a place for ANN prediction. The manual describes the ANN as particularly recommended for complicated structure geometries and variable wave conditions, while also stressing the need for a sufficiently wide and homogeneous database (van der Meer et al., 2018). That is a balanced position: ANN is neither magic nor a gimmick; it is a practical surrogate model whose legitimacy depends on database quality and engineering discipline.

Recent overtopping ML literature has broadened beyond ANN. Later work has compared or used support vector machines, gradient-boosted trees, random forests, Gaussian process regression, and deep learning methods. Some studies report better benchmark performance than older ANN formulations for particular filtered datasets or structural classes, but the broader lesson is not that one algorithm is universally best. The lesson is that feature design, training data filtration, hyperparameter tuning, interpretability, and uncertainty awareness matter as much as the label attached to the algorithm (den Bieman et al., 2021; Alqahtani et al., 2023; Elbisy, 2024; Habib et al., 2025).

Advantages of data-driven overtopping models. They can be fast, scalable, and convenient for repeated screening. They are well suited to batch evaluation, sensitivity studies, preliminary dimensioning, and rapid comparison of alternatives. Once trained, they can return estimates almost instantly for many scenarios.

Limitations that never disappear. They can reproduce dataset biases. They are weaker under extrapolation. They may be less interpretable than direct formulae. They do not replace project-specific physical modelling when the geometry is exceptional or when safety consequences are high. They are also limited by the representation of extreme events in the training data; recent work highlights that high overtopping rates are often underrepresented, which can degrade performance in the very region that matters most for safety (Habib et al., 2025).

The most professional stance is therefore hybrid. Use empirical equations to understand trend direction and order of magnitude. Use the neural-network or machine-learning tool for rapid multi-parameter interpolation. Use diagnostics and range checks to determine whether the estimate is comfortably inside the learned domain. Then decide whether the output is adequate as a first estimate or whether more rigorous follow-up is mandatory.

Strengths and weaknesses of ML overtopping models

- Speed

  - Strength: Very fast after training

  - Weakness / caution: Can encourage overuse without adequate checking.

- Complex geometry

  - Strength: Can learn interactions difficult to compress into one equation

  - Weakness / caution: Still limited to patterns actually represented in training data.

- Batch operation

  - Strength: Excellent for many scenarios and sensitivity studies

  - Weakness / caution: May hide individual outlier cases unless warnings are reviewed carefully.

- Interpretability

  - Strength: Can be improved with physically designed features and post-analysis

  - Weakness / caution: Often weaker than direct closed-form equations.

- Uncertainty

  - Strength: Ensembles and percentiles can express spread

  - Weakness / caution: Spread is not the same as full epistemic certainty.

- Design-stage use

  - Strength: Useful for conceptual design and screening

  - Weakness / caution: Not sufficient alone for final high-consequence design.

## 7. Official EurOtop ANN versus the current custom Python workflow

This distinction is one of the most important messages in the entire document. Users often hear the phrase “overtopping neural network” and assume that every modern Python implementation is simply a direct copy of the official EurOtop ANN. That is not true here. The current code is a custom implementation inspired by the overtopping ML tradition, but it is not the official EurOtop ANN tool as documented in the EurOtop manual.

Official EurOtop ANN characteristics. EurOtop describes a tool based on dimensionless physically motivated inputs. It highlights a network architecture with 14 dimensionless input parameters, a hidden layer of 20 neurons and 1 bias, and output-specific networks for overtopping discharge, reflection, and transmission. The manual further states that the overtopping prediction tool is based on 500 neural networks, whose distribution of outputs yields the mean estimate and the 5%/95% confidence interval (van der Meer et al., 2018).

Current train.py characteristics. The current implementation is built with scikit-learn. It constructs a Pipeline consisting of SimpleImputer(strategy=‘median’), StandardScaler, and MLPRegressor. The saved metadata explicitly identifies the model type as a “Bagged MLPRegressor ensemble on log10(sq) with engineered hydraulic features”. The default configuration uses 50 models, not 500, and the default hidden layers are (192, 96, 48, 24) rather than a single 20-neuron hidden layer.

Official target normalization versus the project target normalization. The official EurOtop discourse typically centres on the classical dimensionless discharge Qclassical=q/gH3. In the corrected script, the internal target is now sq=q/gHm0,toe3. That means the script is now aligned with the classical square-root scaling, while still applying it with toe-based wave conditions and still fitting log10(sq) after floor clipping. The model therefore remains a custom ML implementation, but its target normalisation is now directly consistent with standard overtopping practice.

Official inputs versus the project inputs. The official EurOtop ANN is explicitly based on dimensionless inputs such as Hm0/L, h/L, ht/Hm0, Bt/L, B/L, Rc/Hm0, Ac/Hm0, Gc/L, slope descriptors, and roughness/permeability descriptors. The current train.py starts from a set of 15 base dimensional inputs and then internally engineers dimensionless ratios and other transforms. In practice, that means the custom script still respects the physical logic of overtopping ML, but it does so through a different implementation path.

Official tool uncertainty versus the project tool uncertainty. EurOtop speaks about the distribution created by 500 networks and uses that to produce a 90% confidence interval. The current script produces spread indicators from its own bagged ensemble and reports P05, P50, and P95 from the distribution of ensemble predictions. These are useful spread descriptors, but they should be described carefully as ensemble percentiles in this custom workflow, not as the official EurOtop ANN confidence methodology.

Practical conclusion. The current workflow should be interpreted as a custom overtopping neural-network estimator with strong EurOtop-style physical feature engineering, not as a literal reproduction of the official EurOtop ANN executable. This is not a weakness. It is simply a matter of being exact about what tool is actually being used.

Official EurOtop ANN input logic as described in the EurOtop manual

- Hm0,t/Lm−1,0,t: Wave steepness / breaking context

- β [rad]: Wave obliquity

- h/Lm−1,0,t: Shoaling / depth context

- ht/Hm0,t: Toe submergence

- Bt/Lm−1,0,t: Toe width effect

- db/Hm0,t: Berm level effect

- B/Lm−1,0,t: Berm width effect

- Rc/Hm0,t: Relative crest height

- Ac/Hm0,t: Relative armour crest height

- Gc/Lm−1,0,t: Crest width effect

- cotαd: Downstream slope

- cotαincl: Average run-up/down slope

- D/Hm0,t: Structure permeability / roughness indicator

- γf: Roughness/permeability dissipation factor

At-a-glance comparison

- Model family

  - Reference tool: Official EurOtop ANN

  - Uploaded tool: Custom bagged MLPRegressor ensemble

- Input philosophy

  - Reference tool: Dimensionless physically designed inputs supplied to ANN

  - Uploaded tool: Dimensional base inputs supplied by user, then transformed into engineered features

- Number of networks

  - Reference tool: 500 networks for overtopping spread description

  - Uploaded tool: 50 bagged MLP models by default

- Representative hidden structure

  - Reference tool: 14 inputs -> 20 hidden neurons -> 1 output

  - Uploaded tool: Hidden layers (192, 96, 48, 24) by default

- Target emphasis

  - Reference tool: Classical ANN overtopping prediction context

  - Uploaded tool: log10(sq) with sq=q/(gHm0,toe3)

- Spread outputs

  - Reference tool: Mean with 5% and 95% exceedance / confidence outputs

  - Uploaded tool: Mean plus P05, P50, P95 from the custom ensemble

- Implementation form

  - Reference tool: Dedicated overtopping ANN tool

  - Uploaded tool: Python scikit-learn backend plus Tkinter GUI

## 8. Detailed theory of the train.py model

This section describes the project backend exactly as an engineering model, not just as software. The goal is to show the chain from raw database rows to saved model bundle to predicted overtopping discharge.

### 8.1 Computational objective

The backend predicts mean overtopping for arbitrary user scenarios. Operationally, the final user-facing quantities are q in l/s/m and q in m³/s/m, together with ensemble percentiles and range warnings. Internally, however, the model is trained on a custom non-dimensional target sq and specifically on log10(sq).

The command-line parser itself states the target definition. The script description in the parser is “Neural-network predictor for overtopping discharge using adimensional sq=q/g Hm0,toe3”. That single line deserves serious emphasis, because it defines the entire numerical target space of the model.

Why log-transform the target. Overtopping discharges span many orders of magnitude, from nearly zero overtopping to strongly overtopping cases. Training directly on raw q often leads to a model dominated by the largest values. Training on a logarithm of a dimensionless target compresses the dynamic range and generally stabilises the regression problem. The implementation handles zero and near-zero values by imposing a floor before the logarithm is taken.

### 8.2 Training data filtration and target construction

The database is first reduced to the required columns. The backend requires the 15 base feature columns and the measured discharge column q. Any database row missing these critical requirements cannot participate in training.

The script explicitly filters for valid q and valid Hm0,toe. It retains rows where q is not null, Hm0,toe is not null, Hm0,toe is finite, and Hm0,toe>0. Then it clips q at a lower bound of zero before constructing the target. This means negative measured discharges, if any were present, are not used as negative values in the learning problem.

Target construction in the script is exact and simple. For each valid row, the scale factor is g Hm0,toe3. The raw target is sq=q/g Hm0,toe3. Non-finite target values are discarded. The stored modelling target sq_train is then produced by clipping sq at the configurable floor sq_floor, which defaults to 0.01. The regression target actually passed to the MLP ensemble is ylogsq=log10(sq,train).

Implication of the floor. The floor is not a physical correction to the database. It is a training stabilisation device. Cases with extremely low or zero overtopping are compressed into the lower tail at sq_floor in log-space for the purpose of fitting. In practice, this means the model is explicitly prioritised as a stable regression tool over many orders of magnitude, rather than a literal zero-detection device.

Core target construction used in train.py:

$$

scale&=g Hm0,toe3sq&=qscalesq&=max(sq,0)sq,train&=max(sq,sq,floor)y&=log10(sq,train)

$$

### 8.3 Base features and engineered features

The model does not rely only on the 15 base inputs. It deliberately engineers additional features so the neural network receives ratios and transforms that are closer to the established overtopping physics. This is one of the strongest technical choices in the script. It allows the model to work not only with raw dimensions in metres and seconds, but also with physically interpretable combinations.

Toe wavelength and wave steepness are explicitly computed. From Tm−1,0,toe, the script constructs Lm−1,0,toe=g T2/(2π) and a corresponding L0,toe using the same numerical form. It then computes wave_steepness =Hm0/L0 and sqrt_wave_steepness, which are used to build breaker-style regime features.

Slope cotangents are converted back to tangents where needed. Since many classical overtopping expressions work with tan(α) rather than cot(α), the script computes inverse cotangent values using safe division. Those are then combined with square-root wave steepness to create xi_m10_lower and xi_m10_upper.

The rest of the feature engineering is mostly dimensionless ratio building. This is very much in the spirit of EurOtop-style scaling. Ratios by Hm0 express relative heights and widths compared with wave height. Ratios by Lm−1,0 express relative geometric scales compared with wavelength. Additional interaction terms encode directional effects and combined hydraulic-roughness effects.

Engineered features created inside train.py

- Lm−1,0,toe

  - Computed form: g Tm−1,0,toe2/(2π)

  - Physical purpose: Toe-based wavelength scale

- L0,toe

  - Computed form: g Tm−1,0,toe2/(2π)

  - Physical purpose: Auxiliary wavelength used for wave steepness in the script

- wave_steepness

  - Computed form: Hm0,toe/L0,toe

  - Physical purpose: Compact forcing descriptor

- sqrt_wave_steepness

  - Computed form: max(wave_steepness,0)

  - Physical purpose: Used in breaker-style features

- xi_m10_lower

  - Computed form: tan(αd)/sqrt_wave_steepness

  - Physical purpose: Lower-slope regime indicator

- xi_m10_upper

  - Computed form: tan(αu)/sqrt_wave_steepness

  - Physical purpose: Upper-slope regime indicator

- h_over_Lm10_toe

  - Computed form: h/Lm−1,0,toe

  - Physical purpose: Depth-to-wavelength ratio

- Hm0_toe_over_Lm10_toe

  - Computed form: Hm0,toe/Lm−1,0,toe

  - Physical purpose: Wave steepness-like scaling

- ht_over_Hm0_toe

  - Computed form: ht/Hm0,toe

  - Physical purpose: Toe submergence ratio

- Bt_over_Lm10_toe

  - Computed form: Bt/Lm−1,0,toe

  - Physical purpose: Toe width ratio by wavelength

- B_over_Lm10_toe

  - Computed form: B/Lm−1,0,toe

  - Physical purpose: Berm width ratio by wavelength

- hb_over_Hm0_toe

  - Computed form: hb/Hm0,toe

  - Physical purpose: Berm vertical ratio

- Rc_over_Hm0_toe

  - Computed form: Rc/Hm0,toe

  - Physical purpose: Classical relative freeboard

- Ac_over_Hm0_toe

  - Computed form: Ac/Hm0,toe

  - Physical purpose: Armour freeboard ratio

- Gc_over_Lm10_toe

  - Computed form: Gc/Lm−1,0,toe

  - Physical purpose: Crest width ratio by wavelength

- h_over_Hm0_toe

  - Computed form: h/Hm0,toe

  - Physical purpose: Depth-to-height ratio

- Bt_over_Hm0_toe

  - Computed form: Bt/Hm0,toe

  - Physical purpose: Toe width ratio by wave height

- B_over_Hm0_toe

  - Computed form: B/Hm0,toe

  - Physical purpose: Berm width ratio by wave height

- Gc_over_Hm0_toe

  - Computed form: Gc/Hm0,toe

  - Physical purpose: Crest width ratio by wave height

- freeboard_sum_over_Hm0_toe

  - Computed form: (Rc+Ac)/Hm0,toe

  - Physical purpose: Combined crest-elevation descriptor

- Rc_over_Ac

  - Computed form: Rc/Ac

  - Physical purpose: Relative crest partitioning

- Bt_over_B

  - Computed form: Bt/B

  - Physical purpose: Relative toe/berm width balance

- beta_abs

  - Computed form: |β|

  - Physical purpose: Angle magnitude

- cos_beta

  - Computed form: cos(β)

  - Physical purpose: Directional effect

- sin_beta_abs

  - Computed form: |sin(β)|

  - Physical purpose: Directional effect

- gf_cos_beta

  - Computed form: gf cos(β)

  - Physical purpose: Combined roughness-direction interaction

### 8.4 Learning architecture and ensemble design

The backend uses a scikit-learn Pipeline. Each model in the ensemble is a pipeline with three stages: median imputation, standardisation, and MLPRegressor. This is a sensible configuration for tabular engineering data. Median imputation protects the model from sporadic missing values. Standardisation prevents large-magnitude variables from numerically dominating the optimisation. The MLP then learns a nonlinear regression map in the transformed feature space.

The individual neural network is a multilayer perceptron with ReLU activation and Adam optimisation. In the current defaults, the hidden-layer structure is (192, 96, 48, 24). Early stopping is activated, using a validation fraction of 0.12, with n_iter_no_change = 20. The regularisation coefficient alpha is 0.0003, the initial learning rate is 0.00075, and the maximum iteration count is 10000.

The model is not a single neural network by default. The backend builds an ensemble of n_models networks, with the default set to 10. Each network is fitted on a bootstrap resample if more than one model is requested. This is a bagging strategy. The effect is to reduce variance, stabilise the prediction, and enable percentile outputs across the ensemble.

Why bagging is valuable here. Overtopping datasets are noisy and highly heterogeneous. A single MLP can be sensitive to the exact training split and optimisation path. A bagged ensemble reduces sensitivity to any one resample and creates a practical predictive spread. That spread is not a full probabilistic uncertainty model, but it is a much better operational output than a single point estimate with no spread information.

Pipeline used by each ensemble member:

SimpleImputer(strategy="median")
→ StandardScaler()
→ MLPRegressor(
    hidden_layer_sizes=(192, 96, 48, 24),
    activation="relu",
    solver="adam",
    alpha=0.0003,
    learning_rate_init=0.00075,
    early_stopping=True,
    validation_fraction=0.12,
    n_iter_no_change=20,
    max_iter=10000,
)

### 8.5 Holdout validation and full-data refit

The current workflow uses a two-stage logic that deserves explicit recognition. First, it evaluates model performance on a holdout subset. Second, after that evaluation, it refits the final ensemble on the full valid dataset and saves that refitted model as the production bundle. The GUI documentation correctly describes this distinction and explicitly tells the user that holdout diagnostics indicate validation performance, whereas the final saved model corresponds to the refitted production model.

Why this matters. Many users assume that the model used for diagnostics is the same fitted object later used in production. In the current workflow, that is not exactly the case. The holdout stage answers “How well does the approach generalise on unseen data?” The full-data refit answers “What model do I want to deploy after I have judged the approach acceptable?” Both are valid objectives, but they should not be conflated.

Holdout split details. The default holdout fraction is 15%. The code attempts to create stratification bins from the log-transformed target using quantile cuts, which is a sensible way to prevent the validation subset from being dominated by only one part of the overtopping range. If viable bins are not available, the split can still proceed without stratification.

Saved model metadata records this design choice. The metrics include a flag stating that the saved model was refit on the full dataset. This is excellent practice because it prevents future confusion when a user inspects the diagnostics file months later.

### 8.6 Diagnostics and reported metrics

The diagnostics are not cosmetic. They are the primary evidence that training produced a reasonable model. The backend computes metrics in three spaces: log10(sq), sq, and q in l/s/m. That is a robust reporting choice, because it lets the user judge performance in the transformed fitting space and in physical engineering units.

Reported statistics include R2, MAE, Median AE, RMSE, and bias. In addition, the diagnostics store counts such as training rows, holdout rows, full rows, number of bagged models, hidden layers, test size, number of engineered features, and the fact that the saved model was refit on the full dataset. The feature ranges are also stored in the diagnostics JSON and later reused by the GUI for range checking.

Prediction tables contain both custom non-dimensional and physical outputs. The printed prediction table includes sq_mean, sq_p05, sq_p50, sq_p95, plus the corresponding q estimates in l/s/m, together with a range_warning field. The user should not ignore the range warning. It is one of the most important outputs in the entire workflow.

### 8.7 Reconstruction of physical discharge from the model target

The model is fitted in log-space, but the engineering result is returned in physical discharge units. After predicting log10(sq) for each ensemble member, the script exponentiates to recover sq, then multiplies by g Hm0,toe3 to reconstruct q in m³/s/m, and finally multiplies by 1000 to express the result in l/s/m.

This reconstruction is possible only if Hm0,toe** is finite and positive.** The code explicitly checks that. If Hm0,toe is missing or non-positive, the script cannot recover physical discharge from the dimensionless target, and that scenario is flagged as invalid in the range warnings.

Reconstruction logic used after prediction:

$$

sq&=10log10(sq)qm3/s/m&=sq g Hm0,toe3ql/s/m&=1000 qm3/s/m

$$

### 8.8 Observed training-domain size in the project database

The project pre-filtered database.csv file contains 9,334 rows and 30 columns in total. When the train.py validity filters are emulated exactly at the level of required overtopping rows, all rows remain for modelling after requiring valid q and valid positive Hm0,toe. This number is important because the model’s credibility depends not only on algorithm choice but also on the breadth of usable calibrated data.

Observed ranges do not guarantee local density. A variable can have a very broad minimum-to-maximum range while still being very sparse in some subregions. Therefore the range table later in Appendix E should be interpreted as a necessary but not sufficient condition for safe interpolation. A case lying within the min-max range may still be poorly represented if the surrounding local data density is weak.

Nevertheless, range checking is still valuable. If a scenario lies outside the observed range for one or more inputs, the operator knows immediately that the model is extrapolating. The GUI exposes this check clearly, which is a strong practical feature of the current workflow.

### 8.9 Engineering interpretation of the custom target space

Because the model is trained on sq rather than directly on q, it learns relative overtopping intensity after normalising by wave height. This tends to help transferability across conditions where the absolute discharge changes strongly with Hm0,toe. In effect, the model is asked first to learn how intense overtopping is relative to the wave-height scaling, and only afterwards to reconstruct the dimensional discharge.

This design has two consequences. First, the model is strongly sensitive to the quality of the supplied Hm0,toe value, because the same variable appears in both the inputs and the target normalisation. Second, the physical discharge reconstruction can change rapidly when Hm0,toe changes, even if the predicted sq changes only moderately. Users should therefore treat Hm0,toe as one of the most influential and quality-critical inputs.

This is also why the corrected target is easier to interpret. The script’s internal sq is now directly comparable, in scaling terms, with literature values based on q/g H3, provided the user keeps track of the exact wave-height definition used in the numerator scaling. In this workflow, the reference height is Hm0,toe, so any comparison with published values should respect that same toe-based hydraulic definition.

Critical modelling note. The current workflow uses physically informed feature engineering and data-driven regression, but it does not enforce hard physical monotonicity constraints. If a user performs wide parameter sweeps far from the training domain, some locally counterintuitive trends may still appear. That is a normal risk in flexible ML models and one more reason to inspect the range warning and the spread indicators carefully.

## 9. Detailed practical use of train.py

The project train.py script is the authoritative computational backend. Even when the user works mainly through the GUI, the practical logic still comes from train.py. Understanding it is therefore essential for troubleshooting, automation, and defensible engineering use.

### 9.1 Files produced and consumed by the backend

Core files in the train.py workflow

- database.csv

  - Role: Training database

  - When it is used: Read during model training or auto-training.

- model.joblib

  - Role: Saved trained model bundle

  - When it is used: Loaded during prediction or replaced after retraining.

- diagnostics.json

  - Role: Metrics, metadata, feature ranges, plot paths

  - When it is used: Written after training and read by the GUI.

- plots/…

  - Role: Diagnostic plots

  - When it is used: Written after training to accompany diagnostics.

- predictions.csv

  - Role: Prediction output table

  - When it is used: Written after single or batch prediction.

- input.txt

  - Role: Default batch input file for the GUI and one CLI workflow

  - When it is used: Used for batch prediction when the operator chooses that format.

model.joblib is not just a fitted neural network weight file. It is a complete saved ModelBundle containing the fitted pipelines, the list of feature columns, the feature ranges, metrics, metadata, and target definition information. Operationally, this is beneficial because the prediction step carries its own context with it.

### 9.2 Preparing the environment

For source use, create a dedicated Python environment. The GUI help text recommends a project-specific virtual environment. This is good practice because machine-learning dependencies are sensitive to version mismatches.

A practical Windows sequence is to create the environment, activate it, upgrade pip, install the required packages, and then run the script from the project directory. The GUI help text specifically lists numpy, pandas, matplotlib, scikit-learn, joblib, and pyinstaller among the operational packages for running and packaging the workflow.

py -m venv .venv
.venv\Scripts\activate
py -m pip install --upgrade pip
pip install pyinstaller numpy pandas matplotlib scikit-learn joblib
python train.py --help

### 9.3 Command structure

The script has two top-level subcommands: train and predict. This is intentionally simple. The train mode creates or refreshes a model bundle and diagnostics. The predict mode loads a saved model if it exists, or can auto-train a new model if instructed with a database path.

python train.py train [training arguments]
python train.py predict [prediction arguments]

### 9.4 Training a model

Training is the first operation to run when no model exists or when the database has changed. The minimum required argument is the database path. The model path and diagnostics path have defaults, but in engineering work it is often preferable to state them explicitly for traceability.

The most standard training command is the one documented in the script header itself. This command reads database.csv, fits the model, saves model.joblib, and writes diagnostics.json.

Configurable training arguments. The training subcommand accepts --sq-floor, --n-models, --max-iter, and --random-state in addition to the required database/model/diagnostics paths. These arguments let the operator adjust the floor used before log10(sq), the number of bagged MLP models, the optimisation budget, and the seed controlling reproducibility.

python train.py train --database database.csv --model model.joblib ^
--diagnostics diagnostics.json

What happens internally when this command runs. The script loads the database, filters valid rows, creates the custom target, engineers additional features, performs a holdout split, trains the holdout-stage ensemble, evaluates diagnostics, refits the final ensemble on the full valid dataset, saves the bundle, writes diagnostics, and emits plots.

### 9.5 Understanding the training arguments

Training arguments in train.py

- –database

  - Default: Required

  - Meaning: Path to the training database CSV.

- –model

  - Default: model.joblib

  - Meaning: Path where the saved model bundle will be written.

- –diagnostics

  - Default: diagnostics.json

  - Meaning: Path where metrics, metadata, ranges, and plot paths will be written.

- –sq-floor

  - Default: 0.01

  - Meaning: Lower bound applied before log10(sq).

- –n-models

  - Default: 10

  - Meaning: Number of bagged MLP models in the ensemble.

- –max-iter

  - Default: 10000

  - Meaning: Maximum optimisation iterations for each MLP.

- –random-state

  - Default: 42

  - Meaning: Seed controlling reproducibility and bootstrap sampling.

### 9.6 Single-scenario prediction from the command line

Single-scenario prediction is appropriate when an analyst needs one manually specified case without using the GUI. The script requires the model path and the output path, plus the scenario inputs. The scenario receives a name through --name, which is carried into the output table.

The script exposes all main physical inputs directly as command-line arguments. These include --m, --beta, --h, --hm0-toe, --tm-1-0-toe, --ht, --bt, --gf, --cotad, --cotau, --berm-width, --hb, --rc, --ac, and --gc.

python train.py predict --model model.joblib --output ^
    predictions.csv --name case1 ^
    --m 30 --beta 0 --h 5 --hm0-toe 2.5 --tm-1-0-toe 8 ^
    --ht 5 --bt 0 --gf 0.55 --cotad 2 --cotau 2 ^
    --berm-width 0 --hb 0 --rc 3 --ac 3 --gc 3

Output behaviour. The script prints a compact table to the console and then writes the full prediction table to the CSV specified by --output. The printed compact table includes both the internal sq outputs and the reconstructed q outputs, plus the range warning field.

### 9.7 Batch prediction from input.txt or other scenario files

Batch prediction is the preferred mode when the analyst has many scenarios. The script supports two mutually exclusive batch sources: --from-inp for a pipe-delimited scenario file and --from-csv for a CSV or semicolon-separated file. The GUI also supports broader delimiter-based reading by calling backend helpers, but the CLI is explicit about these two routes.

The --from-inp mode reads a pipe-delimited file with a header. The parser is tolerant to several aliases. For example, it can derive the single roughness factor gf by averaging separate gammaf_d and gammaf_u values if those are present. This is useful when the batch file comes from a format closer to earlier overtopping tools or manually curated operator files.

The --from-csv mode reads a comma-separated or semicolon-separated text file and then normalises common header aliases. If no Name column is present, the script automatically generates scenario names such as scenario_1, scenario_2, and so on.

- python train.py predict –model model.joblib –from-csv scenarios.csv ^ –output predictions.csv

A critical practical point. The batch file must still supply the physical variables expected by the backend. Header aliasing is helpful, but it is not magical. The safest practice is to keep batch columns as close as possible to the canonical variable names.

### 9.8 Auto-training when the model file is missing

Auto-training is a convenience feature, not a substitute for deliberate training review. If the model file named by --model does not exist, the function ensure_bundle() can trigger training automatically, provided that a valid database path is supplied through --database.

This is useful for scripted workflows in which the operator wants predict to be robust to a missing model bundle. However, from a controlled engineering perspective, it is usually better to run explicit training first, review diagnostics, and only then move on to production prediction.

python train.py predict --model model.joblib --database database.csv ^
--from-csv scenarios.csv --output predictions.csv

### 9.9 Meaning of the prediction columns

Columns emitted by the prediction table

- Name: Scenario identifier carried through the workflow.

- sq_mean: Mean predicted custom non-dimensional target sq.

- sq_p05 / sq_p50 / sq_p95: 5th, 50th, and 95th percentile of predicted sq across the ensemble.

- q(l/s/m): Mean predicted overtopping discharge in litres per second per metre.

- q_p05(l/s/m) / q_p50(l/s/m) / q_p95(l/s/m): Ensemble percentile outputs in physical units.

- range_warning: Text description of any out-of-domain base input or missing Hm0,toe issue.

### 9.10 Recommended training and prediction workflow

Step 1. Train explicitly from the chosen database and save both model and diagnostics.

Step 2. Open diagnostics.json and the plots folder; verify that the holdout metrics are reasonable.

Step 3. Preserve the trained model as a versioned artefact if the project requires traceability.

Step 4. Run a small number of known cases first and inspect both the q values and the range warnings.

Step 5. For batch work, check the header names before submitting hundreds of scenarios.

Step 6. After every large batch, scan the warnings column before trusting the results table.

## 10. Detailed practical use of gui.py and gui.exe

The project gui.py script is a Tkinter desktop interface built specifically around the train.py backend. Operationally, the GUI is the easiest way for most engineers to interact with the model because it exposes the prediction, training, batch, diagnostics, and range-check workflows in one desktop window.

The same practical guidance applies to gui.exe if gui.py has been compiled to a Windows executable from the same revision. The executable is simply the packaged delivery form of the same logic. What matters is that the packaged executable and the backend code remain aligned.

### 10.1 What the GUI is designed to do

GUI functional objectives

- Single-case prediction: Enter one scenario manually and obtain mean q plus ensemble spread indicators.

- Batch prediction: Load a scenario file, predict all cases, preview results, and save a CSV.

- Model training / refresh: Train a new model from database.csv and regenerate diagnostics.

- Calibration-domain review: Compare current inputs against min-max ranges stored in the trained model bundle.

- Persistent working state: Restore prior inputs, paths, selected tab, and window geometry from defaults.json.

### 10.2 Startup behaviour

Automatic startup loading is deliberate. The GUI help text states that when the GUI starts, it immediately tries to load the model file shown in the Model file field. If that file exists and is readable, the model is loaded before the user presses any prediction button. This is a convenience feature that turns the desktop app into a near-immediate prediction tool.

If the model does not exist, the GUI still opens in a usable state. The interface remains ready so the user can either train a new model, point the GUI to another existing bundle, or proceed later after preparing the backend files.

Persistent state means the GUI reopens close to the last session. The GUI writes a file called defaults.json and later reads it on startup. Stored values include single-case inputs, model/database/output paths, batch input and output paths, selected notebook tab, and last window geometry. This is extremely useful for iterative engineering work, but it also means the user must remain aware of what prior settings were restored.

### 10.3 File fields used by the GUI

Principal file path controls in the GUI

- Model file: The saved model.joblib bundle used for prediction.

- Database CSV: The source database used when training or refreshing the model.

- Single CSV output: Destination CSV for the latest single-case prediction.

- Diagnostics JSON: Summary file written by the backend after training and later displayed by the GUI.

- Batch input file: Scenario file used in batch prediction; defaults to input.txt at startup.

- Batch output CSV: Destination CSV for the latest batch prediction results.

### 10.4 Single Prediction tab: complete operating procedure

The Single Prediction tab is the primary operator screen. The left-hand side is used for entering or reviewing the current case and the relevant file paths. The right-hand side is used for results interpretation: prediction summary, diagnostics summary, warning panel, and a review table of the current case.

Input grouping is physically meaningful. The GUI groups inputs as Case / Wave, Toe / Slope, and Berm / Crest. That grouping is not merely cosmetic. It helps the operator review the completeness of the hydraulic forcing and geometric definition in an engineering sequence.

Recommended operator sequence. First confirm that the correct model is loaded. Second enter or revise all physical inputs. Third confirm the single output CSV path if you want a saved result. Fourth press Predict q. Fifth read the summary cards carefully. Sixth inspect warnings and the review table. Seventh save the result if needed.

Result interpretation on this tab. The GUI documents the fields clearly: Average q [l/s/m] is the main predicted discharge, P05/P50/P95 [l/s/m] are the spread indicators, Average q [m³/s/m] is the same discharge in SI units, and the warning panel indicates whether the case lies outside the trained domain.

Professional reading of P05, P50, and P95. These should not be treated as guaranteed lower, median, and upper physical truths. They are percentile summaries of the custom ensemble output. Their value lies in telling the user whether the current case is producing a tight or broad ensemble response. Broad spread means the tool itself is less stable for that case, which should lower confidence in blind use of the mean estimate.

Single-case step 1. Review or enter Name, Hm0,toe, Tm−1,0,toe, m, h, and β.

Single-case step 2. Review or enter ht, Bt, gf, cotad, and cotau.

Single-case step 3. Review or enter B, hb, Rc, Ac, and Gc.

Single-case step 4. Confirm model.joblib, database.csv, diagnostics.json, and the single-result output CSV path.

Single-case step 5. Press Predict q.

Single-case step 6. Read the summary cards and the SI conversion field.

Single-case step 7. Inspect the range warning and the row-by-row input review before accepting the result.

Single-case step 8. Press Save single result if a CSV output is required for record keeping.

### 10.5 Batch Prediction tab: complete operating procedure

The Batch Prediction tab is intended for many scenarios at once. The GUI documentation states that accepted formats include pipe-separated text, tab-separated text/TSV, CSV, semicolon-separated text, and files that can be parsed through the backend helpers.

The default startup batch file is input.txt. This is a helpful convention because it gives the operator a known initial path every time the GUI opens. If the current project uses a different batch file, the operator should consciously overwrite or browse away from the default path rather than assuming the field is already correct.

The batch result preview is part of the quality-control workflow. After the run, the table preview shows each scenario name, mean q, P05, P50, P95, and any range warning. This preview is not decorative. It is the operator’s first chance to detect failed input interpretation, obviously incorrect magnitudes, or widespread extrapolation warnings before exporting the final CSV.

Batch step 1. Select the batch input file.

Batch step 2. Confirm or edit the batch output CSV path.

Batch step 3. Press Predict from batch file.

Batch step 4. Inspect the preview table carefully, especially the warning column.

Batch step 5. Use Save batch result to export the latest batch output.

Batch step 6. Use Use default input.txt if you need to restore the standard startup file path.

### 10.6 Training and diagnostics from the GUI

The GUI is not prediction-only. It can call the backend training routine directly. This is important for non-programmer operators who still need to refresh the model after the database changes.

The documented GUI workflow for training is straightforward: select the database, confirm model and diagnostics paths, press Train / refresh model, wait for the backend to complete, then allow the newly trained model to load into the GUI.

The diagnostics summary uses the backend’s two-stage logic. The GUI explicitly distinguishes valid rows, n_holdout, n_full, R2(log10 sq), and R2(q l/s/m). Users must understand that n_holdout refers to the validation subset and n_full refers to the final refit dataset used for the saved production model.

Best practice after retraining is not merely to trust that training completed. The operator should review both the model summary and diagnostics summary and, if possible, the plots referenced by diagnostics.json.

### 10.7 Parameter Ranges tab

This tab is one of the strongest safety features in the interface. It compares the currently entered numeric inputs against the observed training-domain minimum and maximum stored in the loaded model bundle.

Interpretation of statuses. inside means the current value lies within the observed database range. outside means the model is extrapolating in that variable. invalid means the entry is not numerically valid. range unavailable means the saved model does not provide a usable range for that field. model not loaded means no bundle is currently available for comparison.

Why range review should be done before prediction rather than after. If a case is obviously outside the trained domain, the user may choose to adjust the scenario, verify the data entry, or reframe the modelling strategy before obtaining a potentially misleading estimate.

### 10.8 defaults.json and state persistence

defaults.json is operationally convenient but procedurally important. Because it restores prior inputs and file paths, it helps long-running engineering studies. But in regulated or high-consequence work it also means the operator should verify that the restored paths still correspond to the intended current project files.

Reset logic. The GUI documentation states that if the user wants a full reset, the GUI should be closed and defaults.json deleted. The next launch will then restore built-in defaults instead of the last session state.

### 10.9 Running gui.py from source and using gui.exe

Source execution is appropriate when the environment is managed by a technical user or when the backend may still be evolving. Executable use is appropriate when the workflow is being delivered to an end user who should not need to interact with Python directly.

In either case, the engineering workflow is the same. The operator still loads or trains a model, enters a case or selects a batch file, inspects warnings, interprets percentile outputs, and saves results.

python gui.py

## 11. Interpretation of outputs, diagnostics, percentiles, and warnings

A prediction is only as useful as its interpretation. The current workflow returns numbers that look precise, often with several decimal places, but engineering interpretation must remain qualitative and conditional.

Mean q is the operational centre of gravity. For most users, the primary result is the mean overtopping discharge in l/s/m or m³/s/m. This is the quantity used for preliminary comparison against tolerability criteria, conceptual design checks, and ranking of alternatives.

P05, P50, and P95 should be read as spread descriptors of the custom ensemble. They are useful because they immediately tell the operator whether the ensemble members agree closely or not. A narrow spread suggests stable interpolation. A wide spread suggests sensitivity, weak support, or some internal disagreement in the ensemble response. The wider the spread, the more conservative the operator should be in using the mean estimate as a stand-alone number.

The diagnostics metrics should be read in layers. R2(log10 sq) indicates how well the model is fitting in the transformed target space it actually optimises. R2(q l/s/m) indicates how well it represents overtopping in the physical quantity that engineers use. MAE and RMSE indicate typical and more outlier-sensitive error magnitude, while bias indicates whether the model tends to overpredict or underpredict on average.

Holdout metrics are more important than refit metrics for credibility. The refit metrics tell the user how well the final model aligns with the full dataset on which it was refit. The holdout metrics tell the user whether the modelling strategy can generalise to unseen samples. In other words, when judging whether the workflow is trustworthy, the operator should weight holdout metrics more heavily than full-data refit plots.

Range warnings are not optional metadata. If the warning says that one or more variables are outside the observed range, the prediction should be described explicitly as an extrapolation. This is still sometimes useful in screening studies, but the operator should avoid presenting such a result as if it had the same credibility as an in-range interpolation.

The absence of a warning is not proof of local data density. A case can be inside the min-max domain and yet still lie in a sparse corner of the database. Therefore an apparently clean range status is a necessary but incomplete credibility test. Use it as the first gate, not the only gate.

How to read key outputs professionally

- Mean q

  - Naive reading: The exact overtopping discharge

  - Correct engineering reading: The central estimate produced by the trained ensemble under the supplied inputs.

- P05 / P95

  - Naive reading: Guaranteed lower and upper physical bounds

  - Correct engineering reading: Low and high ensemble percentile descriptors; useful for spread, not absolute truth.

- R2

  - Naive reading: The model is accurate everywhere

  - Correct engineering reading: Global fit indicator that must be combined with range review and engineering judgement.

- No range warning

  - Naive reading: Safe result

  - Correct engineering reading: Necessary first check passed; local sparsity may still exist.

- Range warning

  - Naive reading: The result is useless

  - Correct engineering reading: Extrapolation flag; the result may still be informative, but confidence should be downgraded.

## 12. Engineering limitations, applicability, and best practice

Every overtopping predictor is conditional on its database and assumptions. This statement applies to empirical equations, classic ANN tools, XGB tools, and the current custom MLP ensemble alike. It is particularly important for overtopping because the response can change by orders of magnitude across realistic design space and because small changes in geometry may strongly affect run-up and discharge.

The current workflow is best used for conceptual design, comparative screening, and rapid iterative assessment. It is well suited to tasks such as: ranking crest freeboard alternatives; comparing the influence of roughness, berm width, or obliquity; screening many scenarios from a forecast or design matrix; and producing preliminary estimates before committing to more expensive analysis.

The workflow is less appropriate as a sole basis for final design decisions when the case is highly unusual, when the structure has features not well represented in the database, when the project is high-consequence, or when the case sits near extreme overtopping levels where database coverage is usually thinner.

Local engineering judgement remains essential. If a result appears physically implausible relative to known overtopping behaviour, do not hide behind the fact that a neural network produced it. Re-check the inputs. Compare against a representative empirical estimate. Inspect the range status. Review the ensemble spread. Then decide whether the scenario should be treated as a suspect extrapolation or as a prompt for deeper analysis.

Database imbalance matters. Recent machine-learning studies emphasise that larger overtopping discharges are often less well represented than smaller ones. That means that the model can appear globally competent while still being weaker in the extreme upper tail. Engineers should keep that asymmetry in mind whenever the case of interest is a high-consequence near-failure or strong-overtopping case.

The safest operational rule is hybrid verification. For in-range routine cases, a trained ML estimate may be entirely adequate for preliminary decisions. For important design choices, compare the ML estimate against at least one classical formula of the relevant structural family and ask whether the order of magnitude and trend are mutually consistent. For out-of-range or unusual cases, escalate to more rigorous methods.

Best-practice rule 1. Use measured or well-transformed toe conditions, not offshore values passed through without thought.

Best-practice rule 2. Check all units before training or prediction; overtopping tools are extremely sensitive to inconsistent input units.

Best-practice rule 3. Treat Hm0,toe and Rc as quality-critical variables because they dominate scaling and resistance.

Best-practice rule 4. Review the parameter ranges before running production batches.

Best-practice rule 5. Do not report the mean alone when the spread is wide; include the percentile context and note the warning status.

Best-practice rule 6. Preserve model version, diagnostics, and database version together when traceability matters.

Best-practice rule 7. Re-train only when the database has materially changed or when a documented model refresh is intended.

Best-practice rule 8. Keep gui.py and train.py from the same revision so the interface and backend remain schema-compatible.

Final engineering stance. A good overtopping ML workflow is not a replacement for engineering. It is a force multiplier for engineering judgement. It becomes dangerous only when the operator stops checking whether the case lies inside the learned domain and whether the output still respects first-order physical expectations.

## 13. Troubleshooting and operator checklists

### 13.1 Common technical and engineering problems

Troubleshooting matrix

- Prediction cannot reconstruct q

  - Likely cause: Hm0,toe missing, invalid, or <= 0

  - Recommended action: Correct the wave-height input; the script cannot rescale sq to physical discharge without a valid Hm0,toe.

- Many scenarios show range warnings

  - Likely cause: Batch file contains out-of-domain cases or incorrect units

  - Recommended action: Check units, headers, and whether the scenario family is genuinely outside the trained database.

- Model file not found

  - Likely cause: No trained bundle at the specified path

  - Recommended action: Run explicit training or use auto-training with a valid database path.

- Diagnostics look good but one case looks implausible

  - Likely cause: Case may be a sparse in-range point or a bad input combination

  - Recommended action: Cross-check inputs, review spread, compare with a formula-based estimate, and inspect range status carefully.

- Batch results all look identical or nearly identical

  - Likely cause: Header mapping failure or missing input columns leading to NaNs/imputation dominance

  - Recommended action: Re-check the batch file headers and confirm all required variables are populated.

- GUI reopens with wrong paths

  - Likely cause: defaults.json restored a previous session state

  - Recommended action: Correct the fields manually or delete defaults.json for a clean reset.

- Training takes long

  - Likely cause: Large ensemble, many rows, high max_iter

  - Recommended action: Wait for completion, or reduce n_models / max_iter deliberately and document the change.

- Unexpected difference between holdout view and deployed behaviour

  - Likely cause: Saved model is refit on the full valid dataset after validation

  - Recommended action: Interpret this as intended workflow design; use holdout metrics for credibility and full refit for deployment.

### 13.2 Pre-run checklist for train.py

- Confirm that database.csv is the intended current training database.

- Confirm that the database contains the required 15 feature columns plus q.

- Confirm that units are consistent with the rest of the project workflow.

- Choose explicit output names for model.joblib and diagnostics.json if version control matters.

- Decide whether defaults (sq_floor, n_models, max_iter, random_state) are appropriate for the current task.

### 13.3 Pre-run checklist for the GUI

- Confirm that the restored defaults correspond to the current project, not a previous session.

- Confirm the loaded model version before reading any prediction as current.

- Check the Database CSV path before pressing Train / refresh model.

- Check the batch file path before pressing Predict from batch file.

- Review the Parameter Ranges tab before accepting a sensitive result.

### 13.4 Post-run checklist for any prediction

- Read the mean q in both l/s/m and m³/s/m if the interface provides both.

- Inspect P05, P50, and P95 to judge spread.

- Read the range warning text fully.

- Check whether the result magnitude is physically plausible against engineering intuition.

- Save the result table or screenshot the critical summary if traceability is required.

## Appendix A. Formula compendium

This appendix collects the main formulas that are most useful for interpreting the current workflow. The formulas below are presented as a compact engineering reference, not as a substitute for reading the full EurOtop chapters.

Formula sheet

- Spectral significant wave height: Hm0=4 m0
Why it matters: Preferred overtopping wave-height variable at the toe.

- Spectral period: Tm−1,0=m−1m0
Why it matters: Preferred overtopping period variable.

- Toe wavelength: Lm−1,0=g Tm−1,022π
Why it matters: Used for wavelength-based normalisation.

- Wave steepness: s=Hm0L0
Why it matters: Compact wave-shape descriptor.

- Breaker parameter: ξm−1,0=tan(α)Hm0/Lm−1,0
Why it matters: Regime indicator for breaking / surging behaviour.

- Generic overtopping form: qg Hm03=a exp​−b RcHm0c
Why it matters: Weibull-shaped conceptual overtopping relation.

- Smooth-slope breaking-wave context: qg Hm03=0.023 tan(α)γb ξ exp​−2.7 Rcξ Hm0 γb γf γβ γv1.3
Why it matters: Mean-value overtopping context for breaking waves.

- Smooth-slope non-breaking context: qg Hm03=0.09 exp​−1.5 RcHm0 γf γβ γ*1.3
Why it matters: Upper-limit / non-breaking context.

- Custom train.py target: sq=qg Hm0,toe3
Why it matters: Internal target used by the current model.

- Custom train.py training target: y=log10(max(sq,sq,floor))
Why it matters: Regression target used by the MLP ensemble.

- Custom physical reconstruction: q=sq g Hm0,toe3
Why it matters: Recover q in m³/s/m after prediction.

- Litres-per-second conversion: ql/s/m=1000 q
Why it matters: Displayed engineering unit in the GUI summary.

## Appendix B. Glossary of symbols and variables

Glossary

- q: Mean overtopping discharge.

- Hm0,toe: Spectral significant wave height at the toe of the structure.

- Tm−1,0,toe: Spectral mean period at the toe of the structure.

- m: Foreshore slope cotangent in the CLI help.

- β / b: Wave obliquity angle in degrees.

- h: Water depth at the toe/front of the structure.

- ht: Toe water depth.

- Bt: Toe width.

- gf: Roughness/permeability factor.

- cotad: Lower slope cotangent.

- cotau: Upper slope cotangent.

- B: Berm width.

- hb: Berm water depth or berm-level descriptor from the dataset convention.

- Rc: Crest freeboard.

- Ac: Armour crest freeboard.

- Gc: Crest width.

- sq: Custom non-dimensional target in the script: q/g Hm0,toe3.

- P05 / P50 / P95: 5th, 50th, and 95th ensemble percentile outputs.

- n_holdout: Number of rows used for holdout validation diagnostics.

- n_full: Number of rows used in the final full-data refit.

## Appendix C. train.py CLI quick-reference

Command patterns

python train.py train \
  --database database.csv \
  --model model.joblib \
  --diagnostics diagnostics.json

python train.py predict \
  --model model.joblib \
  --output predictions.csv \
  --name case1 ...

python train.py predict \
  --model model.joblib \
  --from-inp input.txt \
  --output predictions.csv

python train.py predict \
  --model model.joblib \
  --from-csv scenarios.csv \
  --output predictions.csv

python train.py predict \
  --model model.joblib \
  --database database.csv \
  --from-csv scenarios.csv \
  --output predictions.csv

Selected CLI arguments

- –m

  - Mode: Single-case predict

  - Meaning: Foreshore slope cotangent

- –beta

  - Mode: Single-case predict

  - Meaning: Wave obliquity in degrees

- –h

  - Mode: Single-case predict

  - Meaning: Water depth at toe/front [m]

- –hm0-toe

  - Mode: Single-case predict

  - Meaning: Hm0,toe at toe [m]

- –tm-1-0-toe

  - Mode: Single-case predict

  - Meaning: Tm−1,0,toe at toe [s]

- –ht

  - Mode: Single-case predict

  - Meaning: Toe water depth [m]

- –bt

  - Mode: Single-case predict

  - Meaning: Toe width [m]

- –gf

  - Mode: Single-case predict

  - Meaning: Roughness/permeability factor [-]

- –cotad

  - Mode: Single-case predict

  - Meaning: Lower slope cotangent [-]

- –cotau

  - Mode: Single-case predict

  - Meaning: Upper slope cotangent [-]

- –berm-width

  - Mode: Single-case predict

  - Meaning: Berm width B [m]

- –hb

  - Mode: Single-case predict

  - Meaning: Berm water depth hb [m]

- –rc

  - Mode: Single-case predict

  - Meaning: Crest freeboard [m]

- –ac

  - Mode: Single-case predict

  - Meaning: Armour crest freeboard [m]

- –gc

  - Mode: Single-case predict

  - Meaning: Crest width [m]

- –from-inp

  - Mode: Batch predict

  - Meaning: Pipe-separated scenario file

- –from-csv

  - Mode: Batch predict

  - Meaning: CSV or semicolon-separated scenario file

- –sq-floor

  - Mode: Train / auto-train

  - Meaning: Lower bound before log10(sq)

- –n-models

  - Mode: Train / auto-train

  - Meaning: Number of bagged MLP models

- –max-iter

  - Mode: Train / auto-train

  - Meaning: Maximum MLP iterations

- –random-state

  - Mode: Train / auto-train

  - Meaning: Random seed

## Appendix D. GUI quick-reference

Main GUI buttons

- Predict q: Runs a single-case prediction using the current model and current inputs.

- Train / refresh model: Fits a new model from the selected database and regenerates diagnostics.

- Save single result: Exports the latest single prediction as CSV.

- Predict from batch file: Runs all scenarios from the selected batch file.

- Save batch result: Exports the latest batch result table as CSV.

- Use default input.txt: Restores the standard startup batch file path.

- Reset defaults: Restores built-in default physical inputs.

- Open ranges tab / Open batch tab: Quick navigation shortcuts to review domain status or batch workflow.

Main GUI tabs

- Single Prediction: Manual case entry and immediate prediction summary.

- Batch Prediction: Many scenarios at once, with preview and CSV export.

- Parameter Ranges: Per-variable inside/outside review against the trained domain.

- Log: Traceability view of what was loaded, trained, predicted, or saved.

## Appendix E. Observed training-domain ranges from the project database

The following range summary reports the observed min, max, and median values after emulating the backend validity filters used for overtopping training rows. It is based on approximately 10,392 valid rows from the project database. Use this appendix as a range reference, not as proof of local data density.

Observed ranges for base features

- m

  - Min: 6

  - Max: 1000

  - Median: 100

- b

  - Min: 0

  - Max: 80

  - Median: 0

- h

  - Min: 0.016

  - Max: 9.32

  - Median: 0.4

- Hm0,toe

  - Min: 0.003

  - Max: 3.765

  - Median: 0.112

- Tm−1,0,toe

  - Min: 0.495

  - Max: 10.64

  - Median: 1.532

- ht

  - Min: 0

  - Max: 7.78

  - Median: 0.3

- Bt

  - Min: 0

  - Max: 10

  - Median: 0

- gf

  - Min: 0.33

  - Max: 1

  - Median: 0.7

- cotad

  - Min: 0

  - Max: 7.07

  - Median: 2

- cotau

  - Min: -5

  - Max: 9.71

  - Median: 1.72

- B

  - Min: 0

  - Max: 8

  - Median: 0

- hb

  - Min: -0.208333

  - Max: 1.175

  - Median: 0

- Rc

  - Min: 0

  - Max: 8.345

  - Median: 0.148

- Ac

  - Min: 0

  - Max: 7.87

  - Median: 0.132

- Gc

  - Min: 0

  - Max: 5.6

  - Median: 0.032

## Appendix F. Parameter-by-parameter operating notes

m. Use m exactly as defined in the project data convention. In the CLI help it is described as the foreshore slope cotangent. Because the script uses m as a base feature, do not silently swap it with another slope descriptor from external tools without verifying the mapping.

β. Enter the wave obliquity in degrees. The backend will derive beta_abs, cos(β), and |sin(β)| from this field. A sign mistake in obliquity can change directional interaction terms even if the magnitude looks reasonable.

h. This is a local depth input, not a generic offshore descriptor. If the project uses transformed toe conditions, keep the depth consistent with those toe conditions.

Hm0,toe. Treat this as one of the most influential inputs in the entire workflow. It appears both as a model input and in the target scaling. Errors in Hm0,toe can therefore distort the prediction twice: once in feature space and once in physical reconstruction.

Tm−1,0,toe. This variable controls the wavelength scaling used to form several engineered features. A period entered in the wrong units or taken from the wrong spectral definition will propagate through many derived ratios.

ht. Use the toe depth consistently with the schematisation adopted in the project. Small changes in toe definition can be important for regime classification and geometric representation.

Bt. Toe width is particularly important in composite geometries. If the toe is effectively absent, represent that intentionally rather than leaving an unknown value.

gf. Do not treat gf as a cosmetic coefficient. It is a roughness/permeability control that can materially affect overtopping. Use a value consistent with the structure type and the project’s adopted convention.

cotad. This lower slope cotangent is used to reconstruct a tangent and build xi_m10_lower. Avoid entering a slope angle where a cotangent is expected.

cotau. This upper slope cotangent is used similarly for xi_m10_upper. Negative or uncommon values deserve special care and should be checked against the geometry convention in the dataset.

B. Berm width should be specified consistently with the project’s schematisation. If no berm is present, use the correct no-berm representation rather than inventing a nominal small value.

hb. The berm vertical descriptor should be entered according to the dataset’s meaning. Because conventions vary between tools, this is a field that deserves explicit review whenever data are exchanged between software packages.

Rc. Crest freeboard is a first-order resistance variable. If Rc is wrong, the overtopping estimate can become grossly misleading even if all other inputs are reasonable.

Ac. Armour crest freeboard provides additional crest-elevation information. It should not be left inconsistent with Rc.

Gc. Crest width affects the horizontal extent of the crest region. Its influence is often secondary to freeboard but still meaningful, especially in composite geometries and post-run-up passage behaviour.

## Appendix G. Example operator workflows

Workflow 1 - First-time setup.

1. Place train.py, gui.py, database.csv, and input.txt in the project directory.

2. Create and activate the Python environment.

3. Run a first explicit training command to create model.joblib and diagnostics.json.

4. Open the GUI only after the first model exists, so the interface can load immediately.

5. Check the ranges and run one known benchmark scenario before production use.

Workflow 2 - Daily GUI use for one scenario.

1. Open gui.py or gui.exe and let it load the last model automatically.

2. Verify that the restored paths in defaults.json still point to the correct project files.

3. Edit the current case in the Single Prediction tab.

4. Review the Parameter Ranges tab.

5. Run Predict q, inspect mean q and percentile spread, and save the result if needed.

Workflow 3 - Batch study with many scenarios.

1. Prepare a batch file with the required headers and units.

2. Open the Batch Prediction tab and confirm the current batch file path.

3. Run the batch prediction and inspect the preview table before export.

4. Scan for widespread warnings or suspiciously identical outputs.

5. Export the batch CSV only after the preview looks credible.

Workflow 4 - Model refresh after database change.

1. Replace or update database.csv deliberately and archive the previous version if traceability matters.

2. Run Train / refresh model from the GUI or train.py train ... from the command line.

3. Review diagnostics.json, holdout metrics, and plots before accepting the new model.

4. Record the updated model file name or folder if the project requires version control.

## Appendix H. Detailed interpretation of diagnostics.json

diagnostics.json is the machine-readable summary of model quality. It stores four broad classes of information: metrics, metadata, feature_ranges, and plot_paths. A disciplined workflow should preserve this file together with the corresponding model bundle, because the model without diagnostics is difficult to audit later.

metrics contains the quantitative fit statistics. In the current backend, these include row counts, R2, MAE, Median AE, RMSE, bias, the target floor, number of models, hidden-layer structure, test size, number of engineered features, and MLP hyperparameters. When a model is refreshed, this section is the fastest way to determine whether the new version is comparable to or better than the previous version.

metadata explains what the model actually is. This includes the database path used, the model type string, the complete list of feature columns, the base feature columns, the target column name, the target definition, and the reported output units. For long-term maintainability, the metadata section is almost as important as the raw scores.

feature_ranges is operationally important because it is what the GUI uses to warn about extrapolation. If a user loads a model bundle from an incorrect project or from an older revision, the range logic in the GUI will also change. That is one more reason to keep project artefacts organised.

plot_paths points to the diagnostic plot files generated under the plots directory. These plots provide visual context for holdout and full-data performance. A metrics-only review is acceptable for quick checks, but a proper model review should also look at the plots.

Most important diagnostics.json fields

- metrics.train_rows / test_rows / holdout_rows / full_rows: How many rows were used in each stage of the workflow.

- metrics.r2_log10_sq: Fit quality in the transformed training space.

- metrics.r2_q_lpsm: Fit quality in physical discharge units.

- metrics.bias_q_lpsm: Average physical overprediction or underprediction tendency.

- metrics.n_models: Number of bagged MLP models used in the ensemble.

- metrics.hidden_layers: Network architecture used in the current model version.

- metadata.model_type: One-line description of the modelling strategy.

- metadata.target_definition: Explicit statement of the internal target; must be checked carefully.

- feature_ranges: Min-max training domain used later by the GUI range checker.

- plot_paths: Locations of the generated plot files.

## Appendix I. Command and usage recipe library

Recipe 1 - Rebuild the model from scratch.

python train.py train ^
    --database database.csv ^
    --model model.joblib ^
    --diagnostics diagnostics.json ^
    --n-models 10 ^
    --max-iter 10000 ^
    --random-state 42

Recipe 2 - Predict one manual case after training.

python train.py predict ^
    --model model.joblib ^
    --output predictions.csv ^
    --name test_case ^
    --m 30 --beta 0 --h 5 --hm0-toe 2.5 --tm-1-0-toe 8 ^
    --ht 5 --bt 0 --gf 0.55 --cotad 2 --cotau 2 ^
    --berm-width 0 --hb 0 --rc 3 --ac 3 --gc 3

Recipe 3 - Predict a batch from the default-style input file.

python train.py predict ^
    --model model.joblib ^
    --from-inp input.txt ^
    --output predictions.csv

Recipe 4 - Predict a CSV batch and auto-train if needed.

python train.py predict ^
    --model model.joblib ^
    --database database.csv ^
    --from-csv scenarios.csv ^
    --output predictions.csv

Recipe 5 - Run the GUI from source.

python gui.py